Report on the UML diagram and XML Schema Definition (XSD) for the newly proposed FWE ADE

Deliverable D2.1

The IN-SOURCE Project is funded by EU Horizon 2020 programme and national funders.

| Project number: | 730254 |
| --- | --- |
| Project acronym: | IN-SOURCE |
| Project title: | INtegrated analysis and modelling for the management of sustainable urban FWE ReSOURCEs |
| Start date of Project: | 2018-06-01 |
| Duration: | 36 month |

| Deliverable reference number and title | D 2.1 – Report on the UML diagram and XML Schema Definition (XSD) for the newly proposed FWE ADE |
| --- | --- |
| Due date of deliverable: | April 2020 |
| Actual submission date: | see “History” Table below |

Short Description:

This document explains the Unified Modelling Language (UML) diagram for the newly proposed Food Water Energy – Application Domain Extension (FWE ADE) of CityGeographicalMarkupLanguage (CityGML v2.0)

Keywords:

Data Model, CityGML ADE, XSD, FWE Nexus

| History |  |  |  |
| --- | --- | --- | --- |
| Version | Author(s) | Comment | Date |
| 1.0 | Rushikesh Padsala (Rushi) | First internal draft | 21.07.19 |
| 2.0 | Rushikesh Padsala (Rushi) | Second draft - UML diagram changes and updated FWE ADE properties | 19.05.20 |
|  | Volker Coors | Review the document v2.0 | 20.5.2020 |
| 2.1 | Rushikesh Padsala (Rushi) | version 2.0 + New chapters (Appendix, References) | 01.06.2020 |
|  | Volker Coors | Approved as a final document | 01.06.2020 |

# Table of Contents

Abbreviated Terms	5

UML Relations Notation	6

Introduction	7

Scope	8

FWE ADE Concept	9

FWE ADE	13

A.	FWEBuilding – An extension to CityGML Building	13

B.	FWELandUse – An extension to CityGML LandUse	21

C.	FWEArea – An extension to CityGML Core	31

D.	FWESystem – An extension to CityGML Core	38

Remarks	45

References	46

Appendix	47

# Abbreviated Terms

FWE 	                    Food Water Energy

OGC                      Open Geospatial Consortium

CityGML	          City Geography Markup Language

ADE	                    Application Domain Extensions

UML               	Unified Modeling Language

XML			Extensible Markup Language

XSD			XML Schema Definition

GIS			Geographical Information System

DLM			Digital Landscape Model

# UML Relations Notation

Multiplicity

| 0 | No instances (rare) |
| --- | --- |
| 0..1 | No instances, or one instance |
| 1 | Exactly one instance |
| 1..1 | Exactly one instance |
| 0..* | Zero or more instances |
| * | Zero or more instances |
| 1..* | One or more instances |
|  | If the multiplicity is not available, by default multiplicity 1 is assumed |

# Introduction

Cities are undergoing rapid expansion throughout the globe. As a result, they face common challenges to provide food, water and energy (FWE) supplies under healthy and economically productive conditions. Decision-makers, such as governments, investors and city developers must be able to understand, quantify and visualise multiple interdependent impacts. To tackle such an issue, IN-SOURCE aims to develop an integrated urban data and modelling framework to help cities analyse and characterise FWE systems and its nexus.

Given the heterogeneity of temporal and territorial entities, such as population, their FWE demands and its impact from LandUse changes which are connected to different spatial levels from individual building level to cities or district level, the adoption of a common, shared and open urban data model is crucial. OGC CityGML is used as the only available free standardised urban data model that allows spatial modelling of semantically different georeferenced objects such as buildings, LandUse and other physical elements of the real world. With its built-in mechanism to extend its core data model with domain-specific objects and attributes (a.k.a Application domain Extension – ADE), a new Food Water Energy ADE (FWE ADE) is proposed in this document.

The FWE ADE is a cohesive data model which allows decision-makers to analyse and visualise the study region relating to FWE and its nexus for any urban areas using geospatial information. Further, this data model can be used to support new simulation interfaces for predicting futuristic FWE nexus scenarios connected to population and LandUse change. For quantitative analysis and visualisation across all the spatial levels of any study area, FWE ADE is attached to buildings, scaling it up to LandUse, further extending it up to neighbourhoods, areas or zones and at the end to the regional level of boroughs, cities and districts.

The structure of this document is as follow: (i) A brief introduction on CityGML, and CityGML ADE is explained under the chapter “Scope”. (ii) Based on “Scope”, the newly proposed FWE ADE and its connection to individual spatial levels are explained under “Concept”. (iii) Based on “Concept”, for each FWE ADE’s spatial level, a UML data model diagram along with its complete list of all the included FWE and its nexus related properties, a possible data source column against each property to find the datasets,  related codelist, details about its XSD schema and some web 3D visualisation examples are explained under “FWE ADE”. (iv) Lastly, few general notes about the current version of FWE ADE are mentioned under “Remarks”.

# Scope

Quantitative analysis of FWE and its nexus is essential information to study infrastructural demand and supply chain for any urban area in sustainable urban development. When required, this information can then further be extended to calculate the Green House Gas emissions while dealing with climate change mitigation and adaptation. To store, structure and visualise such data and indicators, a data model is of extreme importance. Since such a data model is space and time-bounded, introducing a geospatial component to it is inevitable but of non-existent in the present time. With, the OGC approved XML based open data model of CityGML, and its mechanism of further extending its data model with an ADE, developing FWE ADE is made plausible. Already available in the core data model of CityGML are the modules of buildings, LandUse, vegetation, roads, street furniture, bridges, tunnels and water bodies. These modules describe how these individual city elements can be modelled along with its georeferenced geometry and semantics in 4 different levels of details (LoD) from 0 to 4 with appearance textures (optional).

Figure 1: An UML package diagram showing the existing thematical modules of CityGML

To extend the existing CityGML modules with new properties/attributes or to introduce an entirely new thematic module to the core of CityGML, ADEs are used. It essential to note that the CityGML ADEs only stretches the data model of CityGML to fit a particular application needs. It is no way related to data interoperability or data format conversion between CityGML to any other data model/format and vice versa. In a simple term, CityGML ADE sits on top of CityGML to enrich its core data model to accommodate application-specific new objects and their properties.

As of 2018,  44 different CityGML ADEs were already developed supporting various applications (Biljecki et al., 2018). Out of these 44 ADEs, one ADE which comes close to the current topic of FWE is the Energy ADE (Nouvel et al., 2015). Energy ADE is specifically designed to store building energy-related data and indicators. Different urban energy simulators such as CitySim (Robinson et al., 2009), EnergyPlus (Geiger et al., 2019), SimStadt (Nouvel et al., 2015) makes use of Energy ADE to assess building stock energy demand and simulate its refurbishment scenarios. But, since the intention for Energy ADE is only to support building stock energy simulations, no properties related to FWE nexus are available.  As a result, instead of extending Energy ADE further, altogether, a new FWE ADE is proposed in this document. However, within FWE ADE, specific properties that are linked to building energy demand - potential (explained in further chapters) uses the results of SimStadt’s building energy demand - potential workflows.

In the current FWE ADE version, data structure follows a top-down approach with population and landuse driving the data model. This means that all the subsequent FWE and its nexus properties (explained in further chapters), will change based on changes in population and landuse. Indicators and metrics connected to climate change, transportation, GHG emissions, urban heat islands, wind flow - CFD simulations, wind power potential and their effect on FWE and its nexus are excluded at the moment. Still,  based on use cases, they may be introduced in the future version of FWE ADE. ADEs can always be restructured and extended based on needs.

# FWE ADE Concept

The primary objective of FWE ADE is to extend the existing CityGML data model, to store FWE and its nexus related properties for developing a digital twin of the study area. By doing so, not only the status quo can be mapped, analysed and visualised, but future analytical scenarios can also be built, simulated and visualised. For stakeholders, such an integrated data model and visualisation tools can further support them to sketch sustainable urban development policies which is a need of the time.

Keeping in mind the challenges of data availability and its granularity, FWE ADE introduces four thematic modules: “FWEBuilding”, “FWELandUse”, “FWEArea” and “FWESystem”.

Micro Level

Regional Level

Figure 2: FWE ADE Thematic Module at different spatial level

FWEBuilding and FWELandUse extend the existing CityGML data model for building and LandUse by introducing new FWE and its nexus related properties and feature classes. FWEArea and FWESystem are the two new thematic modules which are newly introduced to store FWE and its nexus related data at the spatial level of political boundaries such as neighbourhoods/areas/zones and scaling it up to the system boundary of the entire study area respectively. The two new thematic modules are introduced in the CityGML Core.

Newly defined FWEArea object is defined as a city object having multisurface geometry covering the entire study area. It can be neighbourhood boundaries, zonal boundaries, municipality boundaries, or political boundaries which are typically static do not change with time. Based on the underlying FWEBuilding and FWELandUse, FWE and its nexus related properties along with temporal changes of population and LandUse change are summarised and introduced at this spatial level. Additionally, properties which naturally belongs only at FWEArea level or datasets of certain FWE and its nexus properties which are only available at FWEArea level are introduced here. Similar to FWEArea, FWESystem is also defined as a city object having multisurface covering the entire study area. Extending FWEArea up by a spatial level, FWESystem covers entire system boundary of the study area which can be borough, city or even an entire district. These boundaries are also static and typically do not change with time. Since FWE ADE is an integrated data model of different spatial levels, for any given time and use case, data related to properties of population, FWE and its nexus can easily be scaled up or down from a taken spatial level using data inter or extrapolation.

Figure 3: A UML package diagram showing dependencies for FWE ADE

(FWE ADE modules (orange) dependent on CityGML modules (yellow) and CityGML core (blue))

The FWE ADE deems to act as a data interchange format for workflows in the FWE domain. An example of one such data interchange process describing SimStadt’s biomass potential workflow with the FWE ADE is explained in the appendix at the end of this document.

Within IN-SOURCE, FWE ADE will be applied to the case study regions of Gowanus (Neighborhood in NYC) / Brooklyn (Borough in NYC), Vienna (City in Austria) and Landkreis Ludwigsburg (District in Germany). A few case study regions related geospatial that will be used along with FWE ADE can be found from:

Gowanus:

CityGML Building/Existing - https://www.dropbox.com/home/IN-SOURCE-Gowanus%20Project/Gowanus_Site_Info/2019/CityGML

CityGML Building/BAU2050 - https://www.dropbox.com/home/IN-SOURCE-Gowanus%20Project/Gowanus_Site_Info/2050_BAU/CityGML

CityGML Building/BP2050 - https://www.dropbox.com/home/IN-SOURCE-Gowanus%20Project/Gowanus_Site_Info/2050_BP/CityGML

LandUse Shapefile/ Existing – DoITT/ZoLa (URL will be available soon)

Admin Boundaries Shapefile -  DoITT/ZoLa (URL will be available soon)

System Boundary Shapefile - https://www.dropbox.com/home/IN-SOURCE-Gowanus%20Project/Gowanus_Site_Info/2019?preview=Gowanus_Border_EPSG32118.zip

Brooklyn:

CityGML Building/Existing - https://www.lrg.tum.de/gis/projekte/new-york-city-3d/

LandUse Shapefile/Existing – DoITT/ZoLa (URL will be available soon)

Admin Boundaries Shapefile – DoITT/ZoLa (URL will be available soon)

System Boundary Shapefile – DoITT/ZoLa (URL will be available soon)

Vienna:

CityGML Building/Existing - https://www.wien.gv.at/ma41datenviewer/public/start.aspx

LandUse Shapefile/Existing - https://www.data.gv.at/katalog/dataset/stadt-wien_realnutzungskartierungab200708wien/resource/e644d5b3-7d03-4c7e-b4f2-8204e80b4bc1

Admin Boundaries Shapefile - https://www.data.gv.at/katalog/dataset/stadt-wien_bezirksgrenzenwien

System Boundary Shapefile - https://www.data.gv.at/katalog/dataset/stadt-wien_bezirksgrenzenwien

Landkreis Ludwigsburg:

CityGML Building/Existing - https://www.lgl-bw.de/unsere-themen/Produkte/Geodaten/3D-Gebaeudemodelle/

LandUse Shapefile/Existing - https://www.lgl-bw.de/unsere-themen/Produkte/Geodaten/Digitale-Landschaftsmodelle/

Admin Boundaries Shapefile - https://www.lgl-bw.de/unsere-themen/Produkte/Geodaten/Verwaltungsgrenzen/

System Boundary Shapefile - https://www.lgl-bw.de/unsere-themen/Produkte/Geodaten/Verwaltungsgrenzen/

# FWE ADE

## FWEBuilding – An extension to CityGML Building

The FWEBuilding module of FWE ADE extends the CityGML thematic module of Building with FWE and its nexus related properties. A data model to store semantic georeferenced 3D building geometries along with its properties is already available in the data encoding standards for CityGML. From the existing data model, a particular focus is drawn to the properties: “class”, “function” and “usage” of feature “_AbstractBuilding” which represents an abstract class of CityGML Building. These properties categorise individual buildings based on building typologies. While for the properties: class and function, case study region specific codelist are developed, for the property: usage, a common codelist is adopted for all the case study regions. Codelist of property: usage is based on food-related activities of an individual building (production, consumption and/or distribution) and is derived from the property: function.

Based on these pre-existing properties, the CityGML Building data model is extended further to store FWE and its nexus related domain-specific properties for buildings. A UML diagram for FWEBuilding showing the extended properties of CityGML Building is shown below in figure 4.

As per figure 4, first, the existing CityGML feature of  _AbstractBuilding is extended with additional properties stereotyped as “ADEElement”. Stereotype ADEElement is used to distinguish the newly introduced additional properties against the existing properties of _AbstractBuilding. This feature type is further associated with a new feature type “BuildingSurvey” which is defined as a property: “buildingSurveyProperties” of the ADEElement _AbstractBuilding. BuildingSurvey is used as a cover term to store all the FWE and its nexus related properties in the CityGML Building. Since a building can be surveyed at different year intervals with the minimum being ones, a multiplicity of one to many [1...*] is defined between _AbstractBuilding and BuildingSurvey. For each year interval, BuildingSurvey is composed of multiple “FoodSurvey” feature type based on its property of “foodCategory”, a “WaterSurvey” feature type and an “EnergySurvey” feature type, each defined as properties of “foodSurveyProperties”, “waterSurveyProperties” and “energySurveyProperties” respectively in BuildingSurvey. FoodSurvey, WaterSurvey and EnergySurvey, along with other properties of BuildingSurvey, inherits the property of population and surveyYear from BuildingSurvey. For any individual CityGML building, property:surveyYear enables FWE ADE to store FWE and its nexus related properties for building in different year intervals, including population change.

Figure 4: A UML class diagram for FWEBuilding module of FWE ADE

(yellow box is an existing module of  CityGML core, orange boxes are designed as part of  the FWE ADE)

A complete list of  “FWEBuilding” properties along with its description and code list is available as below:

ADEElement: _AbstractBuilding

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| buildingFootprintArea | building footprint area | double | 1 | SimStadt |
| buildingFootprintAreaUnit | measuring unit of buildingfootprint area | string | 1 | SimStadt |
| heatedArea | area to be heated in a building | double | 0..1 | SimStadt |
| heatedAreaUnit | measuring unit of heatedArea | string | 0..1 | SimStadt |
| totalFloorArea | the total floor area of a building | double | 1 | SimStadt |
| totalFloorAreaUnit | measuring unit for totalFloorArea | string | 1 | SimStadt |
| fweAreaName | the name of the belonging FWEArea | string | 1 | user-defined based on overlying FWEArea |
| fweSystemName | the name of the belonging FWESystem | string | 1 | user-defined based on overlying FWESystem |

FeatureType: BuildingSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| surveyYear | survey year | integer | 1 | user-defined |
| surveyYearVersion | for survey scenarios belonging to the same survey year (e.g. 2020 existing scenario, 2020 business as usual scenario, 2020 best practice scenario) | double | 0..* | user-defined |
| surveyDescription | user description for survey | string | 0..* | user defined |
| population | building population | integer | 0..1 | census/SimStadt |

FeatureType: FoodSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| foodCategory | list of food categories | codelist | 0..* | GIS/literature survey |
| foodDemand | food demand according to foodCategory in a building | double | 0..1 | literature survey/estimation based on building population |
| foodDemandUnit | measuring unit for foodDemand | string | 0..1 | literature survey |
| foodProduction | food production according to foodCategory in a building | double | 0..1 | literature survey/estimation based on building population |
| foodProductionUnit | measuring unit for foodProduction | string | 0..1 | literature survey |
| foodProductionWater Demand | water demand to grow/process foodCategory in a building | double | 0..1 | literature survey/estimation based on building population |
| foodProductionWater DemandUnit | measuring unit for foodProductionWater Demand | string | 0..1 | literature survey |
| foodProductionEnergy Demand | energy demand to grow/process foodCategory in a building | double | 0..1 | literature survey/estimation based on building population |
| foodProductionEnergy DemandUnit | measuring unit for foodProductionEnergy Demand | string | 0..1 | literature survey |
| foodConsumption | food consumption according to foodCategory in a building | double | 0..1 | literature survey/estimation based on building population |
| foodConsumptionUnit | measuring unit for foodConsumption | string | 0..1 | literature survey |

FeatureType: WaterSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| domestic Water Demand | the amount of water required for domestic use (includes all building functions) in a building | double | 0..1 | literature survey/estimation based on building population |
| domestic Water DemandUnit | measuring unit for domesticWaterDemand | string | 0..1 | literature survey |
| domesticHot Water Demand | the amount of hot water required for domestic use (includes all building functions) in a building | double | 0..1 | literature survey/estimation based on building population |
| domesticHot Water Demand Unit | measuring unit for domesticHotWater Demand | string | 0..1 | literature survey |
| domesticHot WaterEnergy Demand | the energy required for producing hot water for domestic use (includes all building functions) | double | 0..1 | SimStadt |
| domesticHot WaterEnergy Demand Unit | measuring unit for domesticHotWaterEnergy Demand | string | 0..1 | SimStadt |
| domestic WasteWater | the amount of domestic waste water produced (includes all building functions) | double | 0..1 | literature survey/estimation based on building population |
| domesticWaste WaterUnit | measuring unit for domesticWasteWater | string | 0..1 | literature survey |
| domestic Solid Waste | the amount of domestic solid waste produced (includes all building functions) | double | 0..1 | literature survey/estimation based on building population |
| domestic Solid WasteUnit | measuring unit for domesticSolidWaste | string | 0..1 | literature survey |
| totalDomestic Waste | the amount of total domestic waste produced (includes all building functions) | double | 0..1 | domesticWasteWater+domestic SolidWaste - GIS |
| totalDomestic WasteUnit | measuring unit for totalDomesticWaste | string | 0..1 | literature survey |

FeatureType: EnergySurvey

| Property | Description | DataType | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| spaceHeatingDemand | the total space heating demand of a building (includes all building functions) | double | 0..1 | SimStadt |
| spaceHeatingDemandUnit | measuring unit for spaceHeatingDemand | string | 0..1 | SimStadt |
| spaceCoolingDemand | the total space cooling demand of a building (includes all building functions) | double | 0..1 | SimStadt |
| spaceCoolingDemandUnit | measuring unit for spaceCoolingDemand | string | 0..1 | SimStadt |
| residentialElectricity Demand | electricity demand if residential building | double | 0..1 | SimStadt |
| residentialElectricity Demand Unit | measuring unit for residentialElectricity Demand | string | 0..1 | SimStadt |
| electricityPotentialFromPV | the total electricity potential from PV panels installed in a building (includes all building functions) | double | 0..1 | SimStadt |
| electricityPotentialFromPV Unit | measuring unit for electricityPotentialFromPV | string | 0..1 | SimStadt |

CodeList: Class (CityGML Building: _AbstractBuilding) for Ludwigsburg

| ID | Description |
| --- | --- |
| 31001 | Gebaeude |
| 51001 | Turm |
| 51002 | Bauwerk oder Anlage für Industrie und Gewerbe |
| 51003 | Vorratsbehälter, Speicherbauwerk |
| 51006 | Bauwerk oder Anlage für Sport, Freizeit und Erholung |
| 51007 | Historisches Bauwerk oder historische Einrichtung |
| 51009 | Sonstiges Bauwerk oder sonstige Einrichtung |

CodeList: Function (CityGML Building: _AbstractBuilding) for Ludwigsburg

http://www.citygmlwiki.org/images/7/7f/BuildingFunctionTypeAdV-trans.xml (ALKIS)

CodeList: Class (CityGML Building: _AbstractBuilding) for Vienna

(will be available soon)

CodeList: Function (CityGML Building: _AbstractBuilding) for Vienna

(will be available soon)

CodeList: Class (CityGML Building: _AbstractBuilding) for New York City

https://www1.nyc.gov/assets/planning/download/pdf/data-maps/open-data/pluto_datadictionary.pdf?r=20v2 (PLUTO Data Dictionary – Pg. C1-C4)

CodeList: Function (CityGML Building: _AbstractBuilding) for New York City

https://www1.nyc.gov/assets/planning/download/pdf/data-maps/open-data/pluto_datadictionary.pdf?r=20v2 (PLUTO Data Dictionary – Pg. C1-C4)

CodeList: Usage (CityGML Building: _AbstractBuilding) - Common

| ID | Description |
| --- | --- |
| 1000 | Food production building |
| 1010 | Food distribution building |
| 1020 | Food consumer building |
| 1030 | Food production+distribution building |
| 1040 | Food distribution+consumer building |
| 1050 | Food production+consumer building |
| 1060 | Food production+distribution+consumber building |

CodeList: foodCategory (FWEBuilding: FoodSurvey) - Common

| ID | Description |
| --- | --- |
| 1000 | Cereals |
| 1010 | Vegetables, Pulses and Roots |
| 1020 | Fruits and Nuts |
| 1030 | Vegetable Oil and Oilseeds |
| 1040 | Sugar and other sweeteners |
| 1050 | Stimulants (coffee, tea, spices, chocolate) |
| 1060 | Meats |
| 1070 | Dairy |
| 1080 | Poultry and Eggs |
| 1090 | Fish and other aquatic products |

Location details of FWEBuilding XML schema file are as below:

| XML namespace identifier | URL will be available soon |
| --- | --- |
| XSD file name | FWEBuilding.xsd |
| Recommended namespace prefix | FWEB |

Some web 3D visualisation example of FWEBuilding module using OGC 3D tiles standard and CesiumJS are shown below:

Figure 5: The CityGML Building model of Marbach (Landkreis Ludwigsburg)

Figure 6: Visualisation of Marbach’s space heating demand using its CityGML Building model

## FWELandUse – An extension to CityGML LandUse

The FWELandUse module of FWE ADE extends the CityGML thematic module of LandUse with FWE and its nexus related properties. A data model to store semantic georeferenced landuse geometries along with its properties is already available in the data encoding standards for CityGML. From the existing data model, a particular focus is drawn to the properties: “class”, “function” and “usage” of feature “LandUse”. These properties categorise individual landuse polygons based on landuse typologies. For the property: class, a common codelist across all the case study region is adopted. For the property: function and use, case study region specific codelist are developed.

Based on these pre-existing properties, the CityGML LandUse data model is extended further to store FWE and its nexus related domain-specific properties for landuse. A UML diagram for FWELandUse showing the extended properties of CityGML LandUse is shown below in figure 5.

As per figure 7, first, the existing CityGML feature of  LandUse is extended with additional properties stereotyped as “ADEElement”. Stereotype ADEElement is used to distinguish the newly introduced additional properties against the existing properties of LandUse. This feature type is further associated with a new feature type “LandUseSurvey” which is defined as a property: “landUseSurveyProperties” of the ADEElement LandUse. LandUseSurvey is used as a cover term to store all the FWE and its nexus related properties in the CityGML LandUse. Since landuse can be surveyed at different year intervals with the minimum being ones, a multiplicity of one to many [1...*] is defined between LandUse and LandUseSurvey. For each year interval, LandUseSurvey is composed of multiple “FoodSurvey” feature type based on its property of “foodCategory”, a “WaterSurvey” feature type and an “EnergySurvey” feature type, each defined as properties of “foodSurveyProperties”, “waterSurveyProperties” and “energySurveyProperties” respectively in LandUseSurvey. FoodSurvey, WaterSurvey and EnergySurvey, along with other properties of LandUseSurvey, inherits the property of population and surveyYear from LandUseSurvey. For any individual CityGML LandUse polygon, property:surveyYear enables FWE ADE to store FWE and its nexus related properties for landuse in different year intervals, including population change.

Figure 7: A UML class diagram for FWELandUse module of FWE ADE

(yellow box is an existing module of  CityGML core, orange boxes are designed as part of  the FWE ADE)

A complete list of  “FWELandUse” properties along with its description and code list is available as below:

ADEElement: LandUse

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| surfaceArea | landuse polygon area | double | 1 | FME |
| surfaceAreaUnit | measuring unit of surfaceArea | string | 1 | FME |
| fweAreaName | the name of the belonging FWEArea | string | 1 | user-defined based on overlying FWEArea |
| fweSystemName | the name of the belonging FWESystem | string | 1 | user-defined based on overlying FWESystem |

FeatureType: LandUseSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| surveyYear | survey year | integer | 1 | user-defined |
| surveyYearVersion | for survey scenarios belonging to the same survey year (e.g. 2020 existing scenario, 2020 business as usual scenario, 2020 best practice scenario) | double | 0..* | user-defined |
| surveyDescription | user description for survey | string | 0..* | user defined |
| population | the total population covered by a landuse polygon based on the overlying FWEBuilding or deriving it from overlying FWEArea | integer | 0..1 | census/GIS |
| buildingFootprintArea | the total building footprint area covered by a landuse polygon based on the overlying FWEBuilding | double | 0..1 | GIS |
| buildingFootprintAreaUnit | measuring unit of buildingFootprintArea | string | 0..1 | GIS |

FeatureType: FoodSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| foodCategory | list of food categories | codelist | 0..* | GIS/literature survey |
| foodDemand | the total food demand for individual food categories covered by a landuse polygon based on overlying FWEBuilding or deriving it from the overlying FWEArea | double | 0..1 | literature survey/estimation based on landuse population/GIS |

| foodDemandUnit | measuring unit for foodDemand | string | 0..1 | literature survey |
| --- | --- | --- | --- | --- |
| foodProduction | the total food production for individual food categories covered by a landuse polygon + total food production from overlying FWEBuilding or deriving it from the overlying FWEArea | double | 0..1 | literature survey/estimation based on landuse population/GIS |
| foodProductionUnit | measuring unit for foodProduction | string | 0..1 | literature survey |
| foodProductionWater Demand | the total food production water demand of individual food categories covered by a landuse polygon + total food production water demand from overlying FWEBuilding or deriving it from the overlying FWEArea | double | 0..1 | literature survey/estimation based on landuse population/GIS |
| foodProductionWater DemandUnit | measuring unit for foodProductionWater Demand | string | 0..1 | literature survey |
| foodProductionEnergy Demand | the total food production energy demand of individual food categories covered by a landuse polygon + total food production energy demand from overlying FWEBuilding or deriving it from the overlying FWEArea | double | 0..1 | literature survey/estimation based on landuse population/GIS |
| foodProductionEnergy DemandUnit | measuring unit for foodProductionEnergy Demand | string | 0..1 | literature survey |
| foodConsumption | the total food consumption per individual food categories of buildings covered by a landuse polygon based on overlying FWEBuilding or deriving it from the overlying FWEArea | double | 0..1 | literature survey/estimation based on landuse population/GIS |
| foodConsumptionUnit | measuring unit for foodConsumption | string | 0..1 | literature survey |

FeatureType: WaterSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| domestic Water Demand | the total domestic water demand of buildings covered by a landuse polygon based on overlying FWEBuilding or deriving it from the overlying FWEArea (includes all building functions) | double | 0..1 | literature survey/estimation based on landuse population/GIS |
| domestic Water DemandUnit | measuring unit for domesticWaterDemand | string | 0..1 | literature survey |
| domesticHot Water Demand | the total domestic hot water of buildings demand covered by a landuse polygon based on overlying FWEBuilding or deriving it from the overlying FWEArea (includes all building functions) | double | 0..1 | literature survey/estimation based on landuse population/GIS |
| domesticHot Water Demand Unit | measuring unit for domesticHotWater Demand | string | 0..1 | literature survey |
| domesticHot WaterEnergy Demand | the total domestic hot water energy demand of buildings covered by a landuse polygon based on overlying FWEBuilding or deriving it from the overlying FWEArea (includes all building functions) | double | 0..1 | GIS |
| domesticHot WaterEnergy Demand Unit | measuring unit for domesticHotWaterEnergy Demand | string | 0..1 | GIS |
| domestic WasteWater | the total domestic waste water demand of buildings covered by a landuse polygon based on overlying FWEBuilding or deriving it from the overlying FWEArea (includes all building functions) | double | 0..1 | literature survey/estimation based on landuse population/GIS |
| domesticWaste WaterUnit | measuring unit for domesticWasteWater | string | 0..1 | literature survey |
| domestic Solid Waste | the total domestic solid waste of buildings covered by a landuse polygon based on overlying FWEBuilding or deriving it from the overlying FWEArea (includes all building functions) | double | 0..1 | literature survey/estimation based on landuse population/GIS |
| domestic Solid WasteUnit | measuring unit for domesticSolidWaste | string | 0..1 | literature survey |

| totalDomestic Waste | the total domestic waste of buildings covered by a landuse polygon based on overlying FWEBuilding or deriving it from the overlying FWEArea (includes all building functions) | double | 0..1 | literature survey/estimation based on landuse population/GIS |
| --- | --- | --- | --- | --- |
| totalDomestic WasteUnit | measuring unit for totalDomesticWaste | string | 0..1 | literature survey |

FeatureType: EnergySurvey

| Property | Description | DataType | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| spaceHeatingDemand | the total space heating demand of buildings covered by landuse polygon (includes all building functions) based on overlying FWEBuilding | double | 0..1 | GIS |
| spaceHeatingDemandUnit | measuring unit for spaceHeatingDemand | string | 0..1 | GIS |
| spaceCoolingDemand | the total space cooling demand of buildings covered by landuse polygon (includes all building functions) based on overlying FWEBuilding | double | 0..1 | GIS |
| spaceCoolingDemandUnit | measuring unit for spaceCoolingDemand | string | 0..1 | GIS |
| residentialElectricity Demand | the total electricity demand of residential buildings covered by landuse polygon based on overlying FWEBuilding | double | 0..1 | GIS |
| residentialElectricity Demand Unit | measuring unit for residentialElectricity Demand | string | 0..1 | GIS |
| electricityPotentialFromPV Buildings | the total PV (electric) potential from buildings covered by landuse polygon (includes all building functions) based on overlying FWEBuilding | double | 0..1 | GIS |
| electricityPotentialFromPV Buildings Unit | measuring unit for electricityPotentialFrom PV Buildings | string | 0..1 | GIS |
| vegetationCoverCategory | land cover category for landuse code: 4000 | CodeList | 0..1 | GIS/ Satellite LandCover map |
| biomassPrimaryEnergy Potential | primary biomass energy potential before converting to a secondary energy carrier | double | 0..1 | SimStadt |

| biomassPrimaryEnergy Potential Unit | measuring unit for biomassPrimaryEnergy Potential | string | 0..1 | SimStadt |
| --- | --- | --- | --- | --- |
| biomassToEnergyWoods | the amount of biomass primary energy potential converted to energy woods | double | 0..1 | SimStadt |
| biomassToEnergyWoods Unit | measuring unit for biomassToEnergyWoods | string | 0..1 | SimStadt |
| biomassToBiogas | the amount of biomass primary energy potential converted to biogas | double | 0..1 | SimStadt |
| biomassToBiogasUnit | measuring unit for biomassToBiogas | string | 0..1 | SimStadt |
| biomassToPlantOil | the amount of biomass primary energy potential converted to plant oil | double | 0..1 | SimStadt |
| biomassToPlantOilUnit | measuring unit for biomassToPlantOil | string | 0..1 | SimStadt |
| biomassToBioethanol | the amount of biomass primary energy potential converted to bioethanol | double | 0..1 | SimStadt |
| biomassToBioethanolUnit | measuring unit for biomassToBioethanol | string | 0..1 | SimStadt |
| biomassToResidualSolid Fuel | the amount of biomass primary energy potential converted to residual solid fuel | double | 0..1 | SimStadt |
| biomassToResidualSolid Fuel Unit | measuring unit for biomassToResidualSolidFuel | string | 0..1 | SimStadt |
| biomassToSolidFuel | the amount of biomass primary energy potential converted to solid fuel | double | 0..1 | SimStadt |
| biomassToSolidFuelUnit | measuring unit for biomassToSolidFuel | string | 0..1 | SimStadt |
| biomassToElectricity Potential | the amount of biomass primary energy potential converted to electricity potential | double | 0..1 | SimStadt |
| biomassToElectricity Potential Unit | measuring unit for biomassToElectricityPotential | string | 0..1 | SimStadt |
| biomassToThermal Potential | the amount of biomass primary energy potential converted to thermal potential | double | 0..1 | SimStadt |
| biomassToThermal Potential Unit | measuring unit for biomassToThermalPotential | string | 0..1 | SimStadt |

CodeList: Class (CityGML LandUse: LandUse) - Common

| ID | Description |
| --- | --- |
| 1000 | Settlement Area |
| 2000 | Open Plots and Vacant Lands |
| 3000 | Traffic |
| 4000 | Vegetation |
| 5000 | Water Body |

CodeList: Function (CityGML LandUse: LandUse) for Ludwigsburg

https://sg.geodatenzentrum.de/web_public/gdz/dokumentation/deu/ATKIS-OK%20Basis-DLM%206_0.pdf (ALKIS)

CodeList: Usage (CityGML LandUse: LandUse) for Ludwigsburg

https://sg.geodatenzentrum.de/web_public/gdz/dokumentation/deu/ATKIS-OK%20Basis-DLM%206_0.pdf (ALKIS)

CodeList: Function (CityGML LandUse: LandUse) for Vienna

| ID | Description |
| --- | --- |
| 1000_1001 | Wohn- u. Mischnutzung (Schwerpunkt Wohnen) |
| 1000_1002 | Geschäfts-, Kern- und Mischnutzung (Schwerpunkt betriebliche Tätlgkeit) |
| 1000_1003 | Industrie- und Gewerbenutzung |
| 1000_1004 | Soziale Infrastruktur |
| 1000_1005 | Technische Infrastruktur/Kunstbauten/Sondernutzung |
| 2000_2001 | Erholungs- und Freitzeltelnrichtugen |
| 3000_3001 | Verkehr |
| 4000_4001 | Landwirtschaft |
| 4000_4002 | Naturraum |
| 5000_5001 | Gewässer inkl. Bachbett |

CodeList: Usage (CityGML LandUse: LandUse) for Vienna

| ID | Description |
| --- | --- |
| 1001_1 | Locker bebautes wohn(misch)geblet |
| 1001_2 | Gartenstadt |
| 1001_3 | Dichtes wohn(misch)geblet |
| 1001_4 | Großvolumlger soltärer wohn(misch)bau |
| 1002_1 | Büro- und Verwaltungsvlertel |
| 1002_2 | Solitäre Handeisstrukturen |
| 1002_3 | Geschäfts-, Kern- und Mischgeblet |
| 1002_4 | Mischnutzung wenig dicht / alter Ortskern |
| 1003_1 | Industrie, prod. Gewerbe, Großhandel Inkl. Lager |
| 1004_1 | Kultur Freizelt, Religion, Messe |
| 1004_2 | Gesundhelt und Einsatzorganisationen |
| 1004_3 | Bildung |
| 1004_4 | Sport und Bad (Indoor) |
| 1005_1 | Miltärische Anlagen |
| 1005_2 | Klaraniage, Deponie |
| 1005_3 | Energieversorgung, Rundfunkanlagen |
| 1005_4 | Wasserversorgung |
| 1005_5 | Transformationsfläche, Baustelle, Martertalgew |
| 2001_1 | Park, Grünaniage |
| 2001_2 | Sport und Bad (Outdoor), Camping |
| 2001_3 | Friedhof |
| 3001_1 | Verkehr |
| 4001_1 | Acker |
| 4001_2 | Weingarten |
| 4001_3 | Gärtnerel, Obstplantagen |
| 4002_1 | Wald |
| 4002_2 | Wiese |
| 5001_1 | Gewässer inkl. Bachbett |

CodeList: Function (CityGML LandUse: LandUse) for New York City

https://www1.nyc.gov/assets/planning/download/pdf/data-maps/open-data/pluto_datadictionary.pdf?r=20v2 (PLUTO Data Dictionary – Pg. D1)

CodeList: Usage (CityGML LandUse: LandUse) for New York City

https://www1.nyc.gov/assets/planning/download/pdf/data-maps/open-data/pluto_datadictionary.pdf?r=20v2 (PLUTO Data Dictionary – Pg. D1)

CodeList: foodCategory (FWELandUse: FoodSurvey) - Common

Same as CodeList: foodCategory (FWEBuilding: FoodSurvey) – Pg. 19

CodeList: vegetationCoverCategory (FWELandUse: EnergySurvey) for Ludwigsburg

Based on https://doi.pangaea.de/10.1594/PANGAEA.893195 (Griffiths et al., 2018)

| ID | Description |
| --- | --- |
| 1001 | Grassland |
| 1002 | Winter Cereals |
| 1003 | Maise |
| 1004 | Winter Rapeseed |
| 1005 | Spring Cereals |
| 1006 | Sugar Beet |
| 1007 | Potato |
| 1008 | Grapevine |
| 1009 | Deciduous Mixed Forest |
| 1010 | Coniferous Forest |

Location details of FWELandUse XML schema file are as below:

| XML namespace identifier | URL will be available soon |
| --- | --- |
| XSD file name | FWELandUse.xsd |
| Recommended namespace prefix | FWEL |

Some web 3D visualisation example of FWELandUse module using OGC 3D tiles standard and CesiumJS are shown below:

Figure 8: The CityGML LandUse model of Marbach

Figure 9: The CityGML Building model of Marbach are overlaid on its CityGML LandUse model

## FWEArea – An extension to CityGML Core

The FWEArea module of FWE ADE is a new module in the CityGML core.

Figure 10: A UML class diagram for FWEArea module of FWE ADE

(blue box shows CityGML core, orange boxes are designed as part of  the FWE ADE)

As shown in figure 11, FWEArea feature type (from hereby called as areas) is defined as a city object having multisurface geometry. This geometry represents the political boundaries covering the entire study area, which are usually static and do not change over time. Few examples of FWEArea geometries can be neighbourhood boundaries of Brooklyn, district boundaries of Vienna or town/municipality boundaries of Landkreis Ludwigsburg.

This feature type is then further associated with a new feature type “Area Survey” which is defined as a property: “areaSurveyProperties” of feature type FWEArea. AreaSurvey is used as a cover term to store all the FWE and its nexus related properties in the CityGML FWEArea. Since areas can be surveyed at different year intervals with the minimum being ones, a multiplicity of one to many [1...*] is defined between feature type FWEArea and AreaSurvey. For each year interval, AreaSurvey is composed of multiple “FoodSurvey” feature type based on its property of “foodCategory”, a “WaterSurvey” feature type and an “EnergySurvey” feature type, each defined as properties of “foodSurveyProperties”, “waterSurveyProperties” and “energySurveyProperties” respectively in AreaSurvey. FoodSurvey, WaterSurvey and EnergySurvey, along with other properties of AreaSurvey, inherits the property of population and surveyYear from AreaSurvey. Property:surveyYear enables FWE ADE to store FWE and its nexus related properties of FWEArea in different year intervals, including population change. This is also of particular importance while analysing temporal landuse change patterns of an area. Along with property:SurveyYear of AreaSurvey, properties:settlementArea, openPlotArea, vegetationArea, waterBodyArea, trafficArea allows to store landuse change data for any given year interval.

A complete list of  “FWEArea” properties along with its description and code list is available as below:

FeatureType: FWEArea

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| surfaceArea | polygon area of taken political boundaries | double | 1 | GIS |
| surfaceAreaUnit | measuring unit of surfaceArea | string | 1 | GIS |
| fweAreaName | the name of the belonging FWEArea | string | 1 | user-defined based on FWEArea |
| fweSystemName | the name of the belonging FWESystem | string | 1 | user-defined based on overlying FWESystem |

FeatureType: AreaSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| surveyYear | survey year | integer | 1 | user-defined |
| surveyYearVersion | for survey scenarios belonging to the same survey year (e.g. 2020 existing scenario, 2020 business as usual scenario, 2020 best practice scenario) | double | 0..* | user-defined |
| surveyDescription | user description for survey | string | 0..* | user defined |
| population | the total population covered by FWEArea based on the underlying FWELandUse or deriving it from census | integer | 0..1 | census/GIS |
| buildingFootprintArea | the total building footprint area based on the FWELandUse | double | 0..1 | GIS |
| buildingFootprintAreaUnit | measuring unit of buildingFootprintArea | string | 0..1 | GIS |
| settlementArea | the total settlement area based on the underlying FWELandUse class code: 1000 | double | 0..1 | GIS |
| settlementAreaUnit | measuring unit of settlementArea | string | 0..1 | GIS |
| openPlotArea | the total open plot area based on the underlying FWELandUse class code: 2000 | double | 0..1 | GIS |
| openPlotAreaUnit | measuring unit of openPlotArea | string | 0..1 | GIS |
| trafficArea | the total traffic area (incl. roads, railways) based on the underlying FWELandUse class code: 3000 | double | 0..1 | GIS |
| trafficAreaUnit | measuring unit of trafficArea | string | 0..1 | GIS |
| vegetationArea | the total vegetation area based on the underlying FWELandUse class code: 4000 | double | 0..1 | GIS |
| vegetationAreaUnit | measuring unit of vegetationArea | string | 0..1 | GIS |
| waterBodyArea | the total water body area based on the underlying FWELandUse class code: 5000 | double | 0..1 | GIS |
| waterBodyAreaUnit | measuring unit of waterBodyArea | string | 0..1 | GIS |

FeatureType: FoodSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| foodCategory | list of food categories | codelist | 0..* | GIS/literature survey |
| foodDemand | the total food demand for individual food categories of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population/GIS |
| foodDemandUnit | measuring unit for foodDemand | string | 0..1 | literature survey |
| foodProduction | the total food production for individual food categories of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population/GIS |
| foodProductionUnit | measuring unit for foodProduction | string | 0..1 | literature survey |
| foodProductionWater Demand | the total food production water demand for individual food categories of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population/GIS |
| foodProductionWater DemandUnit | measuring unit for foodProductionWater Demand | string | 0..1 | literature survey |
| foodProductionEnergy Demand | the total food production energy demand for individual food categories of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population/GIS |
| foodProductionEnergy DemandUnit | measuring unit for foodProductionEnergy Demand | string | 0..1 | literature survey |
| foodConsumption | the total food consumption for individual food categories of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population/GIS |
| foodConsumptionUnit | measuring unit for foodConsumption | string | 0..1 | literature survey |

FeatureType: WaterSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| domestic Water Demand | the total domestic water demand of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population /GIS |
| domestic Water DemandUnit | measuring unit for domesticWaterDemand | string | 0..1 | literature survey |
| domesticHot Water Demand | the total domestic hot water demand of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population /GIS |
| domesticHot Water Demand Unit | measuring unit for domesticHotWater Demand | string | 0..1 | literature survey |
| domesticHot WaterEnergy Demand | the total domestic hot water energy demand of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | GIS |
| domesticHot WaterEnergy Demand Unit | measuring unit for domesticHotWaterEnergy Demand | string | 0..1 | GIS |
| domestic WasteWater | the total domestic wastewater of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population /GIS |
| domesticWaste WaterUnit | measuring unit for domesticWasteWater | string | 0..1 | literature survey |
| domestic Solid Waste | the total domestic solid waste of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population /GIS |
| domestic Solid WasteUnit | measuring unit for domesticSolidWaste | string | 0..1 | literature survey |
| totalDomestic Waste | the total domestic waste of an FWEArea based on underlying FWELandUse or deriving it from the overlying FWESystem | double | 0..1 | literature survey/estimation based on FWEArea population /GIS |
| totalDomestic WasteUnit | measuring unit for totalDomesticWaste | string | 0..1 | literature survey |
| totalDomestic WasteEnergy Potential | the total energy potential from the total domestic waste of an FWE Area based on overlying FWESystem or deriving it from literature survey | double | 0..1 | literature survey/estimation based on totalDomesticWaste/GIS |
| totalDomestic WasteEnergy PotentialUnit | measuring unit for totalDomestic WasteEnergyPotential | string | 0..1 | literature survey |

FeatureType: EnergySurvey

| Property | Description | DataType | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| spaceHeatingDemand | the total space heating demand of buildings in an FWEArea based on underlying FWELandUse | double | 0..1 | GIS |
| spaceHeatingDemandUnit | measuring unit for spaceHeatingDemand | string | 0..1 | GIS |
| spaceCoolingDemand | the total space cooling demand of buildings in an FWEArea based on underlying FWELandUse | double | 0..1 | GIS |
| spaceCoolingDemandUnit | measuring unit for spaceCoolingDemand | string | 0..1 | GIS |
| residentialElectricity Demand | the total electricity demand of residential buildings in an FWEArea based on underlying FWELandUse | double | 0..1 | GIS |
| residentialElectricity Demand Unit | measuring unit for residentialElectricity Demand | string | 0..1 | GIS |
| electricityPotentialFromPV Buildings | the total PV (electric) potential from buildings in an FWEArea based on underlying FWELandUse | double | 0..1 | GIS |
| electricityPotentialFromPV Buildings Unit | measuring unit for electricityPotentialFrom PV Buildings | string | 0..1 | GIS |
| biomassPrimaryEnergy Potential | the primary biomass energy potential of an FWEArea based on underlying FWELandUse | double | 0..1 | GIS |
| biomassPrimaryEnergy Potential Unit | measuring unit for biomassPrimaryEnergy Potential | string | 0..1 | GIS |
| biomassToElectricity Potential | the electricity energy potential from biomass of an FWEArea based on underlying FWELandUse | double | 0..1 | GIS |
| biomassToElectricity Potential Unit | measuring unit for biomassToElectricityPotential | string | 0..1 | GIS |
| biomassToThermal Potential | the thermal energy potential from biomass of an FWEArea based on underlying FWELandUse | double | 0..1 | GIS |
| biomassToThermal Potential Unit | measuring unit for biomassToThermalPotential | string | 0..1 | GIS |

CodeList: foodCategory (FWEArea: FoodSurvey) - Common

Same as CodeList: foodCategory (FWEBuilding: FoodSurvey) – Pg. 19

Location details of FWEArea XML schema file are as below:

| XML namespace identifier | URL will be available soon |
| --- | --- |
| XSD file name | FWEArea.xsd |
| Recommended namespace prefix | FWEA |

A web 3D visualisation example of FWEArea module using OGC 3D tiles standard and CesiumJS is shown below:

Figure 11: A 3D map of domestic waste production in FWEAreas (municipality boundaries)

of  Landkreis Ludwigsburg (dummy dataset)

## FWESystem – An extension to CityGML Core

The FWESystem module of FWE ADE is a new module in the CityGML core.

Figure 13: A UML class diagram for FWESystem module of FWE ADE

(blue box shows CityGML core, orange boxes are designed as part of  the FWE ADE)

As shown in figure 13, FWESystem feature type (from hereby called a system) is defined as a city object having multisurface geometry. This geometry represents a system boundary covering the entire study area, which is usually static and does not change over time. Few examples of FWESystem geometries can be borough boundary of Brooklyn, city boundary of Vienna or district boundary of Landkreis Ludwigsburg.

This feature type is then further associated with a new feature type “System Survey” which is defined as a property: “systemSurveyProperties” of feature type FWESystem. SystemSurvey is used as a cover term to store all the FWE and its nexus related properties in the CityGML FWESystem. Since systems can be surveyed at different year intervals with the minimum being ones, a multiplicity of one to many [1...*] is defined between feature type FWESystem and SystemSurvey. For each year interval, SystemSurvey is composed of multiple “FoodSurvey” feature type based on its property of “foodCategory”, a “WaterSurvey” feature type and an “EnergySurvey” feature type, each defined as properties of “foodSurveyProperties”, “waterSurveyProperties” and “energySurveyProperties” respectively in SystemSurvey. FoodSurvey, WaterSurvey and EnergySurvey, along with other properties of SystemSurvey, inherits the property of population and surveyYear from SystemSurvey. Property:surveyYear enables FWE ADE to store FWE and its nexus related properties of FWESystem in different year intervals, including population change. This is also of particular importance while analysing temporal landuse change patterns of a system. Along with property:SurveyYear of SystemSurvey, properties:settlementArea, openPlotArea, vegetationArea, waterBodyArea, trafficArea allows to store landuse change data for any given year interval.

A complete list of  “FWESystem” properties along with its description and code list is available as below:

FeatureType: FWESystem

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| surfaceArea | polygon area of taken political boundaries | double | 1 | GIS |
| surfaceAreaUnit | measuring unit of surfaceArea | string | 1 | GIS |
| fweSystemName | the name of the belonging FWESystem | string | 1 | user-defined based on FWESystem |

FeatureType: SystemSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| surveyYear | survey year | integer | 1 | user-defined |
| surveyYearVersion | for survey scenarios belonging to the same survey year (e.g. 2020 existing scenario, 2020 business as usual scenario, 2020 best practice scenario) | double | 0..* | user-defined |
| surveyDescription | user description for survey | string | 0..* | user defined |
| population | the total population covered by FWESystem based on the underlying FWEArea or deriving it from census | integer | 0..1 | census/GIS |
| buildingFootprintArea | the total building footprint area based on the FWEArea | double | 0..1 | GIS |
| buildingFootprintAreaUnit | measuring unit of buildingFootprintArea | string | 0..1 | GIS |
| settlementArea | the total settlement area based on the underlying FWEArea | double | 0..1 | GIS |
| settlementAreaUnit | measuring unit of settlementArea | string | 0..1 | GIS |
| openPlotArea | the total open plot area based on the underlying FWEArea | double | 0..1 | GIS |
| openPlotAreaUnit | measuring unit of openPlotArea | string | 0..1 | GIS |
| trafficArea | the total traffic area (incl. roads, railways) based on the underlying FWEArea | double | 0..1 | GIS |
| trafficAreaUnit | measuring unit of trafficArea | string | 0..1 | GIS |
| vegetationArea | the total vegetation area based on the underlying FWEArea | double | 0..1 | GIS |
| vegetationAreaUnit | measuring unit of vegetationArea | string | 0..1 | GIS |
| waterBodyArea | the total water body area based on the underlying FWEArea | double | 0..1 | GIS |
| waterBodyAreaUnit | measuring unit of waterBodyArea | string | 0..1 | GIS |

FeatureType: FoodSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| foodCategory | list of food categories | codelist | 0..* | GIS/literature survey |
| foodDemand | the total food demand for individual food categories of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population/GIS |
| foodDemandUnit | measuring unit for foodDemand | string | 0..1 | literature survey |
| foodImport | the total food import for individual food categories in an FWESystem based on literature survey | double | 0..1 | literature survey |
| foodImportUnit | measuring unit for foodImport | string | 0..1 | literature survey |
| foodProduction | the total food production for individual food categories of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population/GIS |
| foodProductionUnit | measuring unit for foodProduction | string | 0..1 | literature survey |
| foodProductionWater Demand | the total food production water demand for individual food categories of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population/GIS |
| foodProductionWater DemandUnit | measuring unit for foodProductionWater Demand | string | 0..1 | literature survey |
| foodProductionEnergy Demand | the total food production energy demand of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population/GIS |
| foodProductionEnergy DemandUnit | measuring unit for foodProductionEnergy Demand | string | 0..1 | literature survey |
| foodConsumption | the total food consumption for individual food categories of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population/GIS |
| foodConsumptionUnit | measuring unit for foodConsumption | string | 0..1 | literature survey |

FeatureType: WaterSurvey

| Property | Description | Data Type | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| domestic Water Demand | the total domestic water demand of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population /GIS |
| domestic Water DemandUnit | measuring unit for domesticWaterDemand | string | 0..1 | literature survey |
| domesticHot Water Demand | the total domestic hot water demand of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population /GIS |
| domesticHot Water Demand Unit | measuring unit for domesticHotWater Demand | string | 0..1 | literature survey |
| domesticHot WaterEnergy Demand | the total domestic hot water energy demand of an FWESystem based on underlying FWEArea | double | 0..1 | GIS |
| domesticHot WaterEnergy Demand Unit | measuring unit for domesticHotWaterEnergy Demand | string | 0..1 | GIS |
| domestic WasteWater | the total domestic wastewater of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population /GIS |
| domesticWaste WaterUnit | measuring unit for domesticWasteWater | string | 0..1 | literature survey |
| domestic Solid Waste | the total domestic solid waste of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population /GIS |
| domestic Solid WasteUnit | measuring unit for domesticSolidWaste | string | 0..1 | literature survey |
| totalDomestic Waste | the total domestic waste of an FWESystem based on underlying FWEArea or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on FWESystem population /GIS |
| totalDomestic WasteUnit | measuring unit for totalDomesticWaste | string | 0..1 | literature survey |
| totalDomestic WasteEnergy Potential | the total energy potential from the total domestic waste of an FWESystem or deriving it from the literature survey | double | 0..1 | literature survey/estimation based on totalDomesticWaste/GIS |
| totalDomestic WasteEnergy PotentialUnit | measuring unit for totalDomestic WasteEnergyPotential | string | 0..1 | literature survey |

FeatureType: EnergySurvey

| Property | Description | DataType | Multiplicity | Possible Data Source |
| --- | --- | --- | --- | --- |
| spaceHeatingDemand | the total space heating demand of buildings in an FWESystem based on underlying FWEArea | double | 0..1 | GIS |
| spaceHeatingDemandUnit | measuring unit for spaceHeatingDemand | string | 0..1 | GIS |
| spaceCoolingDemand | the total space cooling demand of buildings in an FWESystem based on underlying FWEArea | double | 0..1 | GIS |
| spaceCoolingDemandUnit | measuring unit for spaceCoolingDemand | string | 0..1 | GIS |
| residentialElectricity Demand | the total electricity demand of residential buildings in an FWESystem based on underlying FWEArea | double | 0..1 | GIS |
| residentialElectricity Demand Unit | measuring unit for residentialElectricity Demand | string | 0..1 | GIS |
| electricityPotentialFromPV Buildings | the total PV (electric) potential from buildings in an FWESystem based on underlying FWEArea | double | 0..1 | GIS |
| electricityPotentialFromPV Buildings Unit | measuring unit for electricityPotentialFrom PV Buildings | string | 0..1 | GIS |
| biomassPrimaryEnergy Potential | the primary biomass energy potential of an FWESystem based on underlying FWEArea | double | 0..1 | GIS |
| biomassPrimaryEnergy Potential Unit | measuring unit for biomassPrimaryEnergy Potential | string | 0..1 | GIS |
| biomassToElectricity Potential | the electricity energy potential from biomass of an FWESystem based on underlying FWEArea | double | 0..1 | GIS |
| biomassToElectricity Potential Unit | measuring unit for biomassToElectricityPotential | string | 0..1 | GIS |
| biomassToThermal Potential | the thermal energy potential from biomass of an FWESystem based on underlying FWEArea | double | 0..1 | GIS |
| biomassToThermal Potential Unit | measuring unit for biomassToThermalPotential | string | 0..1 | GIS |

CodeList: foodCategory (FWESystem: FoodSurvey) - Common

Same as CodeList: foodCategory (FWEBuilding: FoodSurvey) – Pg. 19

Location details of FWESystem XML schema file are as below:

| XML namespace identifier | URL will be available soon |
| --- | --- |
| XSD file name | FWESystem.xsd |
| Recommended namespace prefix | FWES |

Some web 3D visualisation example of FWESystem module using OGC 3D tiles standard and CesiumJS are shown below:

Figure 12: The FWESystem model of Landkreis Ludwigsburg

Figure 13: A temporal food consumption profile of  Landkreis Ludwigsburg (dummy dataset)

# Remarks

- All the CityGML datasets used with the FWE ADE should be according to the OGC conformance requirements for CityGML. Conformance requirements are the set of rules and regulations for modelling any city object in CityGML. These requirements are documented in the encoding standards of CityGML.

- All the properties of the FWE ADE having multiplicity [1] are mandatory.

- All the stationary energy demand - potential calculations will be done using SimStadt at the FWEBuilding level. Then it will be scaled up to the FWELandUse, FWEArea, and at FWESystem level.

- SimStadt calculates space heating and cooling demands of buildings. Energy demand for appliances in buildings or machinery in industries are not calculated

- Electricity demand is only estimated for residential buildings in the current state of SimStadt.

- Minimum requirements to calculate space heating/cooling demand and electricity demand using SimStadt are CityGML buildings in LoD1 and CityGML building property: function and year of construction.

- Additionally, to calculate PV potential (electricity) of buildings using SimStadt, CityGML buildings in LoD2 is minimum.

- Biomass potential calculation will be done using SimStadt at the FWELandUse level, and then it will be scaled up to the FWEArea and FWESystem level. CityGML LandUse dataset having properties: class, function, use along with the FWELandUse properties: surfaceArea and vegetationCropCover are mandatory.

- In the current state of the FWE ADE, due to data availability, all the food and water-related parameters will be first integrated at the FWESystem level and then scaled down to the FWEArea level. Scaling further down to the FWEBuilding and FWELandUse level may be done based on use cases/scenarios. 3DcityDB software, along with PostgresSQL/PostGIS, will be investigated and extended to support the FWE ADE.

- Within the FWE ADE, all the food related parameters are based on food category codelist.

- The property of domestic waste includes food consumption waste. Food production waste (agricultural waste) are excluded in the current state of the FWE ADE.

- Based on the use cases/scenarios, it is always possible to extend the ADEs further.

# References

- Biljecki, F., Kumar, K. & Nagel, C. (2018). CityGML Application Domain Extension (ADE): overview of developments. Open geospatial data, softw. stand. 3, 13.

- Geiger, A., Benner, J., Häfele, K. & Hagenmayer, V. (2019). Building Energy Simulations at Urban Scale Based on Standardized Data Models Using a Transparent Enrichment Process. In the 16th IBPSA International Conference and Exhibition

- Griffiths, P., Nendel, C., Hostert, P. (2018): National-scale crop- and land-cover map of Germany (2016) based on imagery acquired by Sentinel-2A MSI and Landsat-8 OLI. PANGAEA

- Nouvel, R., Kaden, R., Bahu, J., Kämpf, J., Cipriano, P., Lauster, M., Joachim, B., Munoz, E., Tournaire, O. & Casper, E. (2015). Genesis of the CityGML Energy ADE. In the proceedings of CISBAT international conference.

- Robinson, D., Haldi, F., Kämpf, J., Leroux, P., Perez, D., Rasheed, A., Wilke, U. (2009). CitySim: Comprehensive Micro-Simulation of Resource Flows for sustainable urban planning. Proceedings of BS2009.

- Nouvel, R., Brassel, K-H, Bruse, M., Duminil, E., Coors, V., Eicker, U., Robinson, D. (2015). SimStadt, a new workflow-driven urban energy simulation platform for CityGml city models. CISBAT International Conference 2015. Lausanne.

# Appendix

- SimStadt’s biomass workflow interface with the FWE ADE

As mentioned above in the section “FWE ADE Concept”, the FWE ADE will be used as a data interchange format for the simulation workflows in the FWE nexus domain. One such simulation workflow of calculating biomass potential using SimStadt for an urban region using DLM data and FWE ADE is explained here.

Figure 14: A conceptual sketch of using FWE ADE with SimStadt biomass potential workflow

As shown in the above diagram, as a first step, using FME software,  CityGML LandUse dataset will be enriched with the required input parameters of surfaceArea, surfaceAreaUnit and vegetationCoverCategory available in the FWE ADE XSD (as defined in the parameters list under chapter B). surfaceAreas can be calculated using FME while vegetationCoverCateogry is custom codelist which is based on land cover satellite images. For the German case study of Landkreis Ludwigsburg, vegetationCoverCategory is derived from the 2016 national scale crop and land cover map of Germany based on imagery acquired by Sentinel-2A and Landsat-8 (Griffiths et al., 2018). These input parameters are mandatory for calculating biomass potential from the CityGML LandUse datasets using SimStadt. The enriched CityGML LandUse is then inputted to the SimStadt’s biomass workflow. The output CityGML LandUse file with output parameters from the FWE ADE XSD will be directly added to the 3DCityDB which is already extended to store the FWE ADE parameters. From 3DCityDB, based on the user needs either the enriched CityGML LandUse file with biomass potential can be extracted or the enriched CityGML FWEArea/FWESystem file with biomass potential values can also be extracted.

Figure 15: Visualisation of the Marbach’s biomass potential using enriched CityGML LandUse

¤¤¤¤¤¤¤¤¤¤¤¤¤¤¤¤

https://www.ogc.org/standards/citygml

http://www.citygmlwiki.org/index.php/CityGML_Energy_ADE

http://www.simstadt.eu/de/index.jsp

https://www.ogc.org/standards/3DTiles

https://cesium.com/cesiumjs/

https://portal.opengeospatial.org/files/?artifact_id=47842